AUTOMATED IRRIGATION SYSTEM

A virtual simulation-based irrigation control system that bridges Proteus hardware simulation with a web-based GUI for real-time monitoring and control.

Prerequisites

- VSPE (Virtual Serial Port Emulator)](https://www.eterlogic.com/Products.VSPE.html)

- Proteus 8 Professional](https://www.labcenter.com/)

Setup Guide

- Virtual COM Port Setup

Install **VSPE**

Launch VSPE and:

Go to “Device” → “Create New Device”

Select “Virtual Pair” connector type

Set  “First port”: ‘COM1’, “Second port”: ‘COM2’

Click “Create Device”

- Proteus Simulation

Open ‘ProteusCirccuit.pdsprj’ (located in ‘/Proteus Simulation’ directory)

Configure virtual terminal:

Right-click on "GSM Module" → Edit Properties

Set “Port”  to ‘COM1’ or ‘COM2’ (matching VSPE)

Run simulation

- Start application (Windows)

run.bat

System Workflow

Proteus Simulation

- Soil moisture/temperature sensors generate virtual data

- Data transmitted via virtual COM port (COM1 → COM2)

Web Server

- Listens on COM2 for serial data

- Processes sensor values using Python scripts

- Serves real-time updates via WebSocket

Dashboard

- Displays live sensor values

- Shows irrigation pump status

- Visualizes historical trends

Usage Instructions

Adjust Sensors in Proteus

- Double-click sensor components

- Modify analog values:

- Soil Moisture: 0-1023 (0 = dry, 1023 = wet)

Temperature: 0-100 (℃)

Monitor Web Dashboard

- Access http://localhost:3000

- Real-time updates will appear in 2-3 seconds

Trigger Irrigation

- Click "Manual Override" in dashboard

- Observe pump activation in Proteus simulation

Troubleshooting

| Issue | Solution |
| --- | --- |
| COM Port Conflict | Close other serial port software |
| No Data in Dashboard | Check VSPE connection status |
| Proteus Simulation Freeze | Reduce simulation speed to 75% |
| Web Interface Not Loading | Reload browser |